How do I upgrade the firmware for my ADSL Router TD-W8901G/TD-8816/TD-8817/TD-8840T/TD-8841T?

Step 1 Please go to TP-Link website and download the latest Turkish firmware for TD-W8901G and then extract the files to a folder;

Step 2 Open the web browser and type the IP address of the device in the address bar (default is 192.168.1.1). Press Enter.

Step 3 Type the username and password in the login page, the default username and password both are admin, Then click OK to log into the device.

Step 4 In the setup page of the modem, click on Bakım-> Yazılım, and click Browse to locate the firmware file, and then click Güncellestirme to start the upgrade procedure.

Step 5 Wait for the upgrade completed and the modem will reboot.

Note:

- DO NOT upgrade over wireless connection, it may cause failure to upgrade and damage the modem.

- Make certain that the power supply is stable and can not be cut off during the upgrade.